Design Requirements

| Operator’s eye level above berth | Shall be comfortable for smooth operation and full vision inside deck. |  |
| --- | --- | --- |
| Wheel load maximum | 35 tones including dynamic effect (operating/storm) |  |
| Max. Corner Load | 275 tones including dynamic effect (operating/storm) |  |
| Stability Factor (min.) | 1.3 (under operating condition) 1.1 (under non-operating condition when free on wheels) |  |
| Ambient Temperature for design consideration | 45 deg C (for mechanical components) 50 deg C (for electrical system) |  |
| Humidity | Varies from 23% to 100% |  |
| Source of External Power supply (Incoming) | 415 V; 3phase; 50 Hz AC |  |
| Short circuit level at substation | 50 KA. |  |
| Permissible Voltage and Frequency variations For LT 415 V | Voltage Frequency +10% & - 10% +1% steady and 2% transient |  |
| Rail span | 10 M |  |
| Portal Height | Min. 4.5 M (For easy passage of vehicle) |  |
| Cable Length (m) | 300 M |  |
| Wind load Design wind velocity | The equipment shall be capable of performing all loading / unloading operations for wind up to 18 m / sec wind velocity. The equipment structure shall be stable, free on wheels during wind pressures corresponding to basic wind velocity of up to 24 m/sec as per IS 875 Part III. However, anchoring device and suitable anchor points on the equipment are to be provided, as stand-by for holding the equipment during storm condition and basic wind velocity up to 39 m/sec. |  |
| Drives | Type | Speeds |
| Hoisting & lowering | AC- Sq. Cage Motor with VVVF drive controls. | Hook 20T= 20 m/min Hook 50T= 10 m/min |
| Slew motion | AC- Sq. Cage Motor with VVVF drive controls. | 0-0.5 RPM |
| Boom luffing | Screw / Wire rope system | 0-20 m/min |
| Gantry travel | AC- Sq. Cage Motor with VVVF drive controls. | 0-30 /min |

For Single Jib ELL crane:

| Quantity | 1 No. |
| --- | --- |
| Materials to be handled | ELL crane suitable for handling machine parts, equipment’s for maintenance of vessels and related operations, general cargo supplies to the vessels. |
| Minimum working radius | 9 to 15m |
| Maximum Outreach | Not less than 35m |
| Lifting capacity between any range of 9 to 15m radius | 50T |
| Minimum lifting capacity @ 35m radius (maximum outreach) | 20T |
| Overall height restriction of ELL crane from CD in parking position | 47.802m |
| Overall height restriction from CD when in operation | 47.802m |
| Height of lift at minimum outreach of 9-15m radius | Not less than 35m |
| Height of lift at outreach of 25m | Not less than 25m |
| Height of lift @ maximum outreach of 35m | Not less than 14m |
| Height of lift below deck level | Not less that (-)15m |
| Rail span | 10m |
| Portal clearance | 4.5m |
| Rail size | CR 100 |
| Height of Crane | The maximum height of the cranes during parking position shall be limited to 47.22 m above Mean Sea Level / 47.802 above Port Chart Datum. |

*The above specifications are the minimum requirements.

# For Double Jib ELL crane:

| Quantity | 1 No. |
| --- | --- |
| Materials to be handled | ELL crane suitable for handling machine parts and equipment’s for maintenance of vessels and related operations. |
| Minimum working radius | 9 to 17m |
| Maximum Outreach | Not less than 35m |
| Lifting capacity between any range of 9 to 17m radius | 50T |
| Minimum lifting capacity @ 35m radius (maximum outreach) | 20T |
| Overall height restriction of ELL crane from CD in parking position | 47.802m |
| Overall height restriction from CD when in operation | 47.802m |
| Height of lift at minimum outreach of 9-17m radius | Not less than 22m |
| Height of lift @ maximum outreach of 35m | Not less than 22m |
| Height of lift below deck level | Not less that (-)15m |
| Rail span | 10m |
| Portal clearance | 4.5m |
| Rail size | CR 100 |
| Height of Crane | The maximum height of the cranes during parking position shall be limited to 47.22 m above Mean Sea Level / 47.802 above Port Chart Datum. |

*The above specifications are the minimum requirements