Budgetary Offers are invited for:

1. Supply, Installation, Testing & Commissioning of 630 KVA, 11 KV/433 V, DYn11,Dry Cast Resin type Transformer as per IS 2026/ IS 11171,  with Copper Winding, Tap Settings, Air Cooled, LV & HV Cable Boxes.

Qty: 1 No.

2. Supply, Installation, Testing & Commissioning of Common Metering Panel with  Copper Busbars, Bus duct for direct connection to the DT, RYB Indications, DT Metering arrangements with 1 no. CT operated Smart Meter as per CoPA Standards with a Set of  4 nos. of Class 0.5S  CTs in sealed chamber, Incomer 1 no. 4 Pole EDO ACB 1000, 50 kA, Microprocessor based with O/C+E/F Shunt Trip + U/V Trip, Outgoing 4 nos. 250 A 4 Pole MCCB, 3 nos. 100 A 4 Pole MCCB, Provision for 1 no. 100 A 4P MCCB, and providing individual metering arrangements including Set of  4 nos. of Class 0.5S  CTs with CT operated Smart Meter as per CoPA Standards  in each cubicles.

Qty: 1 No.

The Amount shall be quoted without GST, and GST shall indicate separately.

The conditions may also include.

The Budgetary Offers may be addressed to:

The Asst. Engineer (Ele.)

Electrical Division

Mechanical Engg. Dept.

Cochin Port Authority

W/Island, Kochi -682009