BOQ  for the Revamping of  40T MHC ( Hook / Grab)  – YoM 2012- Make Italgru- Italy, with NEW PLC Logic system for the control  and operation of the crane .  Existing drives ( Hoist/Travelling/ Skew/ Luff( Cylinder)  are of hydraulic dive controlled from Main hydraulic pump attached to Diesel engine/ Electric motor with external supply .

The MHC is used for handling  Dry bulk, breakbulk, machineries , Steel sheets/ coils etc.The details of existing system is also attached .OEM of  crane control system is requested to give offer.

BOQ/ Schedule

| Sl.no | Scope of works | QTY | Rate in Rs. | Amount in Rs. |
| --- | --- | --- | --- | --- |
| 1 | Supply of Items - Supply of PLC CPU panel with tentatively 250-No’s of DI/DO’s at E-Room and at Operator cabin - Supply of Master module which will communicate to NEW / existing slave stations via existing Slip-ring arrangement as required - Supply of Operator chair with console and master controller. - Supply of 2-No’s HMI at E Room & at Operator cabin. Supply of Communication cables inside the crane for communication of PLC modules. - Supply of Communication cables inside the crane for communication of PLC modules. - Supply of 3-No’s of Absolute Encoder - Supply of 2-No’s of angle sensor. - Wired Remote control interface optional to operator cabin operation. - Modules required if any or equivalent interface Modules. - Any type of Field instruments, sensors, Wind speed sensors, Load cell & controller etc. | SET |  |  |
| 2 | - Scope of Services: - Design, Engineering, Manufacturing, Software development, erection, testing, & Commissioning PLC Panel at E Room & Operator cabin station. - The crane will be handed over to User with overall functional test, safety interlock test and endurance test acceptable to the User comparative to the crane present performance. - Engineering, Commissioning of HMI station at E Room & at Operator cabin - Engineering, Commissioning of 3-No’s of Absolute Encoder. - Engineering, Commissioning of 2-No’s of Slew angle sensor - Engineering, Commissioning of Operator cabin console with chair. - Communication Establishment and PLC programming of existing slaves at Lower leg. - Removal of existing system and erection of supplied system. - Post order FEI will study the machine prepare the FDS and will submit the FDS for approval. | SET |  |  |
| 3 | Supply Documents We will supply Three sets of drawings (A4) and Manuals of the entire documentation. - Preliminary documentation - Final documentation for the installation - As built documentation In the details it includes: - Single line diagram - Devices list - Electrical drawings and terminals - Cables list - Description, user, operating, and maintenance instruction | SET |  |  |
| 4. | Dismantling the existing control systems and panels not needed for the new system as per design. | SET |  |  |

Total

*GST / Customs duty etc. if any shall be shown extra.

Note : 1. The items mentioned is tentative and subjected to change as per actual.

2.  For  electro Mechanical works support  will be done by Cochin Port .

3. Delivery and commissioning - 4 months.